STEPS FOR PROGRAMMING:-

Changing Z to C drive and directory to 8086

z:\>  mount c c:\

z:>   c:

c:\>  cd 8086

c:\8086>

Editor: This is used to open the editor for entering code

Step1: c:\8086> masm filename.asm

Step2: Linking: This is used to link the asm file to obj file

c:\8086> link filename.obj

Step3: The obj file is executed to generate the executable file

c:\8086> debug filename.exe

-g: This is used for entire program execution

-t: line by line execution

-u: unassemble the code

-q: To exit the debugger

-d<address of segment>:0

Eg: d 076b:0

Basic 8086 Program Structure:

Any 8086 program consists of two parts:

1. Directives (For the assembler)

2. Instruction and Values (For the compiler)

PART-A: EXPERIMENTS ON 8086 MICROPROCESSOR

Experiment No. 01: Programs for 16-bit arithmetic operations using Various Addressing Modes.

A) ADDITION OF TWO 8BIT NNUMBERS:

REGISTERS USED:AL, BL

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code

code segment

start:mov al,02h

mov bl,03h

add al,bl

int 03h

code ends

end start

OUTPUT:

B) SUBTRACTION OF TWO 8BIT NUMBERS:

REGISTERS USED:AL, BL

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code

code segment

start:mov al,05h

mov bl,03h

sub al,bl

int 03h

code ends

end start

OUTPUT:

C) MULTIPLICATION OF TWO 8BIT NUMBERS:

REGISTERS USED:AL, BL

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code

code segment

start:mov al,05h

mov bl,02h

mul bl

int 03h

code ends

end start

OUTPUT:

D) DIVISION OF TWO 8BIT NUMBERS:

REGISTERS USED:AL, BL

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code

code segment

start:mov al,7h

mov bl,2h

div bl

int 03h

code ends

end start

OUTPUT:

E) ADDITION OF TWO 16BIT NNUMBERS:

REGISTERS USED:AX, BX

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code

code segment

start:mov ax,2345h

mov bx,5623h

add ax,bx

int 03h

code ends

end start

OUTPUT:

F) SUBTRACTION OF TWO 16BIT NUMBERS:

REGISTERS USED:AX, BX

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code

code segment

start:mov ax,7896h

mov bx,2345h

sub ax,bx

int 03h

code ends

end start

OUTPUT:

G) MULTIPLICATION OF TWO 16BIT NUMBERS:

REGISTERS USED:AX,BX

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code

code segment

start:mov ax,2345h

mov bx,3456h

mul bx

int 03h

code ends

end start

OUTPUT:

H) DIVISION OF TWO 16BIT NUMBERS:

REGISTERS USED:AX, BX

SOFTWARE USED: MASM.

assume cs:code

code segment

start:mov ax,4000h

mov bx,2000h

div ax

int 03h

code ends

end start

OUTPUT:

ADDITION OF NUMBERS USING ADDRESSING MODES:

REGISTERS USED:AX,BX,SI,DI

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code,ds:data

data segment

input dw 0F232h,4535h

output dw 0000h

data ends

code segment

start:mov ax,data

mov ds,ax

mov si,offset input

mov di,offset output

mov ax,[si]

add si,0002h

mov bx,[si]

add ax,bx

mov [di],ax

int 03h

code ends

end start

OUTPUT:

Experiment No. 02: Program for sorting an array for 8086.

A)SORTING AN ARRAY OF UNSIGNED INTEGERS IN ASCENDING ORDER

REGISTERS USED:AX, DS, SI, CX, DX

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code,ds:data

data segment

arr1 db 53h,25h,19h,02h,29h

count equ 04h

data ends

code segment

start:mov ax,data

mov ds,ax

mov dx,count

back0:mov cx,count

mov si,offset arr1

bacl1:mov al,[si]

cmp al,[si+1]

jb next

xchg [si+1],al

xchg [si],al

next:inc si

loop back1

dec dx

jnz back0

int 03h

code ends

end start

OUTPUT:

B) SORTING AN ARRAY OF UNSIGNED INTEGERS IN DESCENDING ORDER:

REGISTERS USED:AX, DS, SI, CX, DX

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code,ds:data

data segment

arr1 db 57h,26h,78h,98h,67h

count equ 04h

data ends

code segment

start:mov ax,data

mov ds,ax

mov dx,count

back0:mov cx,count

mov si,offset arr1

back1:mov al,[si]

cmp al,[si+1]

ja next

xchg [si+1],al

xchg [si],al

next:inc si

loop back1

dec dx

jnz back0

int 03h

code ends

end start

OUTPUT:

C) SORTING AN ARRAY USING DATAWORD:

REGISTERS USED:AX, DS, SI, CX, DX

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code,ds:data

data segment

arr1 dw 3242h,8567h,7865h,4536h,9867h

count equ 0004h

data ends

code segment

start:mov ax,data

mov ds,ax

mov dx,count

back0:mov cx,count

mov si,offset arr1

back1:mov ax,[si]

cmp ax,[si+2]

jb next

xchg [si+2],ax

xchg [si],ax

next:add si,0002h

loop back1

dec dx

jnz back0

int 03h

code ends

end start

OUTPUT:

D) SORTING AN ARRAY OF SIGNED INTEGERS IN ASCENDING ORDER:

REGISTERS USED:AX, DS, SI, CX, DX

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code,ds:data

data segment

arr1 db +53h,-25h,-19h,+02h

count equ 04h

data ends

code segment

start:mov ax,data

mov ds,ax

mov dx,count-1

back0:mov cx,dx

mov si,offset arr1

back1:mov al,[si]

cmp al,[si+1]

jl next

xchg [si+1],al

xchg [si],al

next:inc si

loop back1

dec dx

jnz back0

int 03h

code ends

end start

OUTPUT:

E) SORTING AN ARRAY OF SIGNED INTEGERS IN DESCENDING ORDER:

REGISTERS USED:AX, DS, SI, CX, DX

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code,ds:data

data segment

arr db +53h,-25h,-19h,+02h

count equ 04h

data ends

code segment

start:mov ax,data

mov ds,ax

mov dx,count-1

back0:mov cx,dx

mov si,offset arr

back1:mov al,[si]

cmp al,[si+1]

jg next

xchg [si+1],al

xchg [si],al

next:inc si

loop back1

dec dx

jnz back0

int 03h

code ends

end start

OUTPUT:

Experiment No. 03: Program for searching for a number or character in a string for 8086

A) PROGRAM TO FIND WHETHER A CHARACTER IS PRESENT IN STRING OR NOT

REGISTERS USED:AX, DS, SI, CX, DX

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code,ds:data

data segment

str1 db "VNRVJIET$"

str2 db "character is found$"

str3 db "character is not found$"

data ends

code segment

start:mov ax,data

mov ds,ax

mov si,offset str1

mov cx,08h

mov al,"J"

back:cmp al,[si]

je last1

mov dx,offset str2

mov ah,09h

int 21h

jmp last2

last1:mov dx,offset str3

mov ah,09h

int 21h

last2:int 3h

code ends

end start

OUTPUT:

Experiment No. 04: Program for string manipulations for 8086

A) PROGRAM TO MOVE A SOURCE STRING TO DESTINATION

REGISTERS USED:AX, DS, SI, CX, ES, DI

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code,ds:data,es:extra

data segment

str1 db "VNRVJIETECE$"

len1 dw($-str1)

data ends

extra segment

str db 10 dup(?)

extra ends

code segment

start:mov ax,data

mov ds,ax

mov ax,extra

mov es,ax

cld

mov si,offset str1

mov di,offset str

mov cx,len1

dec cx

rep movsb

int 3h

code ends

end start

OUTPUT:

B) PROGRAM TO CHECK WHETHER A CHARACTER IS PRESENT IN STRING OR NOT

REGISTERS USED:AX, DS, ES, SI, CX, DI

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code,ds:data,es:extra

data segment

str2 db "character is available$"

str3 db "character is not available$"

data ends

extra segment

str1 db "VNRVJIET$"

extra ends

code segment

start:mov ax,data

mov ds,ax

mov ax,extra

mov es,ax

mov di,offset str1

mov al,'E'

mov cx,0008h

CLD

REPNE SCASB

JZ next1

next2:mov dx,offset str3

mov ah,09h

int 21h

jmp next3

next1:mov dx,offset str2

mov ah,09h

int 21h

next3:int 03h

code ends

end start

OUTPUT:

C) PROGRAM TO CONCATENATE TWO STRINGS

REGISTERS USED:AX, DS, ES, SI, CX, DI

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code,ds:data,es:extra

data segment

str1 db "VNRVJIET$"

len1 dw($-str1)

str2 db "ELECTRONICS$"

len2 dw($-str2)

data ends

extra segment

str db 15 dup(?)

extra ends

code segment

start:mov ax,data

mov ds,ax

mov ax,extra

mov es,ax

cld

mov si,offset str1

mov di,offset str

mov cx,len1

dec cx

rep movsb

mov si,offset str2

mov cx,len2

rep movsb

int 3h

code ends

end start

OUTPUT:

D) PROGRAM TO REVERSE A STRING.

REGISTERS USED:AX, DS, SI, CX, DI, ES

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code,ds:data,es:extra

data segment

str1 db "VNRVJIET$"

len1 dw($-str1)

data ends

extra segment

str db 15 dup(?)

extra ends

code segment

start:mov ax,data

mov ds,ax

mov ax,extra

mov es,ax

cld

mov si,offset str1

mov di,offset str

mov cx,len1

dec cx

add di,cx

mov byte ptr es:[di],"$"

dec di

back:cld

lod sb

std

sto sb

loop back

int 3h

code ends

end start

OUTPUT:

E) PROGRAM TO COMPARE TWO STRINGS.

REGISTERS USED: AX, DS, SI, CX, DI, ES

SOFTWARE USED: MASM.

PROGRAM:

assume cs:code,ds:data

data segment

str3 db 'strings are equal$'

str4 db 'strings are not equal$'

str1 db 'VNRVJIET$'

data ends

extra segment

str2 db 'VNRVJEET$'

extra ends

code segment

start:mov ax,data

mov ds,ax

mov ax,extra

mov es,ax

mov di,offset str2

mov si,offset str1

mov cx,0008h

cld

repe cmpsb

jz next1

next2:mov dx,offset str4

mov ah,09h

int 21h

jmp next3

next1:mov dx,offset str3

mov ah,09h

int 21h

next3:int 3h

code ends

end start

OUTPUT:

F) ; Program to find length of the string

ASSUME CS:CODE, DS:DATA

DATA SEGMENT

ARI DB VNRVJIET$'

RESULT DB 1 DUP(?)

DATA ENDS

CODE SEGMENT

START:

MOV AX,DATA

MOV DS,AX

MOV SI,OFFSET ARI

MOV DX,0000H

MOV AL,'$'

BACK: CMP AL[SI]

JE NEXT

INC DX

INC SI

JMP BACK

NEXT: MOV RESULT, DX

INT 03H

CODE ENDS

END START

G) PROGRAM TO FIND LARGEST IN A GIVEN DATA OF STRINGS.

data segment

STRING1 DB 08h,14h,05h,0Fh,09h

res db ?

data ends

code segment

assume cs:code, ds:data

start: mov ax, data

mov ds, ax

mov cx, 04h

mov bl, 00h

LEA SI, STRING1

up:

mov al, [SI]

cmp al, bl

jl nxt

mov bl, al

nxt:

inc si

dec cx

jnz up

mov res,bl

int 3

code ends

end start

H)Deleting the substring put from the string computer.

assume cs code,ds: data

data segment

str1 db "computer$"

len dw ($-str1)

str db 20 dup(?)

data ends

code segment

start:mov ax, data

mov ds,ax

mov si,offset str1

mov di,offset str

mov cx, len mov bl,'p'

back:mov al, [si]

cmp al,bl

je next

mov [di], al

inc si

inc di

dec cx

jnz back

imp next1

mov di,offset str

mov cx, len mov bl,'p'

back:mov al, [si]

cmp al,bl

je next

mov [dil,al

inc si

inc di

dec cx

jnz back

jmp next1

next:add si,03

dec cx

jnz back

next1: mov ah, 09h

int 21h

int 03h

code ends

end start

Experiment No. 05: Program to define and call a subroutine which calculates the average of three numbers

REGISTERS USED:AX, DS, SI, BL, DI

SOFTWARE USED: MASM.

PROGRAM

assume cs:code,ds:data

data segment

arr1 db 05h,06h,07h

avg db 03h

data ends

code segment

start:mov ax,data

mov ds,ax

mov si,offset arr1

mov di,offset avg

call average

int 03h

average:mov cx,02h

xor ax,ax

back:mov al,[si]

add [si+1],al

jnc next

inc ah

next:inc si

loop back

mov al,[si]

mov bl,[di]

div bl

ret

code ends

OUTPUT:

INTERFACING MICROPROCESSOR WITH ADC, DAC, STEPPER MOTOR:

STEPS FOR EXCEUTION:

C:\masm 123>masm filename

C:\masm 123>link filename

C:\masm 123>promview

PROMVIEW SOFTWARE:

Select:

1)File operations :Select Format

Press 0-Binary (EXE)

2)File operations:Read

Filename.exe:

Default 0000

3)RAM operation:View

Starting address:starting address+200

:4200

Ending address  :4300

Press ESC key

4)File operation:Select format

Press 1:intel hex

5)File operation: Write

RAM starting address:4200

RAM ending address:4230

Enter file name:ADC.Hex

Exit: Back  to DOS

Are you sure(Y/N):Y

Are you saving file(Y/N):Y

C:\masm 123>edit filename. Hex

Post prom view steps:

C:\masm 123>tc serial 1

Press ‘S’ in kit keyword

UIK-86

Type-‘L’ OFFSET=0

Enter filename: adc.hex

Running the program:

The program can be run using two commands.

i)D4200: Unassembles and displays line wise code.

ii)G4200: It is used to run the program.

ADC INTERFACING:

In interfacing an ADC to the 8086 microprocessor in lab,we have a fixed set of port address.

PA: FF50H                         PC: FF54H

PB: FF52H                         CONTROL WORD REGISTER: FF56H

PORTA-Input port

PORTC Upper: SOC Signal

INTERFACING ADC WITH 8086:

REGISTERS USED:AL,DX,CX

SOFTWARE USED: MASM, promview

PROGRAM:

code segment

assume cs:code

org 2000h

mov dx,0ff56h

mov al,90h

out dx,al

mov dx,0ff54h

mov al,0ffh

out dx,al

mov al,00h

out dx,al

mov al,0ffh

out dx,al

call delay

call delay

mov dx,0ff50h

in al,dx

int 3h

delay:mov cx,0ffffh

loop1:nop

nop

dec cx

jnz loop1

ret

code ends

end

OUTPUT:

| ANALOG I/P | DIGITAL O/P |
| --- | --- |
| 1.2 | 42 |
| 1.7 | 5C |
| 2.1 | 71 |
| 2.5 | 87 |

INTERFACING DAC WITH 8086:

i)PROGRAM TO GENERATE A SQUARE WAVEFORM:

REGISTERS USED:AL,DX,CX

SOFTWARE USED: MASM,promview

PROGRAM:

code segment

assume cs:code

org 2000h

mov dx,0ff56h

mov al,80h

out dx,al

mov dx,0ff52h

next:mov al,0ffh

out dx,al

call delay

mov al,00h

out dx,al

call delay

jmp next

delay:mov cx,07ffh

x:nop

nop

dec cx

jnz x

ret

code ends

end start

OUTPUT:

ii)PROGRAM TO GENERATE A RAMP WAVEFORM:

REGISTERS USED:AL,DX,CX

SOFTWARE USED: MASM,promview

PROGRAM:

code segment

assume cs:code

org 2000h

mov dx,0ff56h

mov al,80h

out dx,al

mov dx,0ff52h

mov al,00h

next:out dx,al

inc al

jmp next

int 3h

code ends

end start

OUTPUT:

INTERFACING STEPPER MOTOR WITH 8086

REGISTERS USED:AL, DX, CX

SOFTWARE USED: MASM, promview

PROGRAM:

mov dx,ff26h

mov al,80h

out dx,al

Step:mov dx,0ff20h

mov al,80h

out dx,al

call delay

mov al,A0

out dx,al

call delay

mov al,E0

out dx,al

call delay

mov al,C0

out dx,al

call delay

jmp step

delay:mov bx,0010

last:mov al,ff

last1:nop

nop

nop

nop

dec al

jnz last1

dec bx

jnz last

ret

OUTPUT:

PART B: EXPERIMENTS ON ARM DEVELOPMENT BOARDS:

Experiment No. 01: Programs to perform arithmetic operations

PROGRAM:

;;; Directives

STACK_ADDR_123G EQU 0x20008000

PRESERVE8

THUMB

; Vector Table Mapped to Address 0 at Reset

; Linker requires __Vectors to be exported

AREA    RESET, DATA, READONLY

EXPORT  __Vectors

__Vectors

DCD  0x20001000  ; stack pointer value when stack is empty

;The processor uses a full descending stack.

;This means the stack pointer holds the address of the last

;stacked item in memory. When the processor pushes a new item

;onto the stack, it decrements the stack pointer and then

;writes the item to the new memory location.

DCD  Reset_Handler  ; reset vector

ALIGN

; The program

; Linker requires Reset_Handler

AREA    MYCODE, CODE, READONLY

ENTRY

EXPORT Reset_Handler

Reset_Handler

;;;;;;;;;;User Code Starts from the next line;;;;;;;;;;;;

MOV R0, #4

MOV R1, #2

START

ADD R3, R1, R0

MUL R2,R1,R0

SUB R4,R0,R1

UDIV R5,R0,R1

B  START

END; End of the program

OUTPUT:

Experiment No. 02: Control ON/OFF of LEDs using switches involving delays.

PROGRAM:

#define SYSCTL_RCGCGPIO_R (*(( volatile unsigned long *)0x400FE608 ) )

#define GPIO_PORTF_DATA_R (*(( volatile unsigned long *)0x40025038 ) )

#define GPIO_PORTF_DIR_R  (*(( volatile unsigned long *)0x40025400 ) )

#define GPIO_PORTF_DEN_R  (*(( volatile unsigned long *)0x4002551C ) )

#define GPIO_PORTF_CLK_EN  0x20

#define GPIO_PORTF_PIN1_EN 0x02

#define LED_ON1 	   0x02

#define GPIO_PORTF_PIN2_EN 0x04

#define LED_ON2		   0x04

#define GPIO_PORTF_PIN3_EN 0x08

#define LED_ON3 	   0x08

#define DELAY_VALUE	  1000000

unsigned long j=0;

void Delay(void){

for (j=0; j<DELAY_VALUE ; j++);

}

int main ( void )

{

volatile unsigned long ulLoop ;

SYSCTL_RCGCGPIO_R |= GPIO_PORTF_CLK_EN ;

ulLoop = SYSCTL_RCGCGPIO_R;

GPIO_PORTF_DIR_R |= GPIO_PORTF_PIN1_EN ;

GPIO_PORTF_DEN_R |= GPIO_PORTF_PIN1_EN ;

GPIO_PORTF_DIR_R |= GPIO_PORTF_PIN2_EN ;

GPIO_PORTF_DEN_R |= GPIO_PORTF_PIN2_EN ;.0

GPIO_PORTF_DIR_R |= GPIO_PORTF_PIN3_EN ;

GPIO_PORTF_DEN_R |= GPIO_PORTF_PIN3_EN ;

while (1)

{

GPIO_PORTF_DATA_R &= LED_ON3;

GPIO_PORTF_DATA_R &= LED_ON2;

GPIO_PORTF_DATA_R |= LED_ON1;

Delay();

GPIO_PORTF_DATA_R &= LED_ON1;

GPIO_PORTF_DATA_R &= LED_ON2;

GPIO_PORTF_DATA_R |= LED_ON3;

Delay();

GPIO_PORTF_DATA_R &= LED_ON3;

GPIO_PORTF_DATA_R &= LED_ON1;

GPIO_PORTF_DATA_R |= LED_ON2;

Delay();

}

}

OUTPUT:

Experiment No. 03: Controlling an LED using switch by polling method/Interrupt method.

i)INTERRUPT METHOD:

PROGRAM:

#include "TM4C123.h"

int main(void)

{

SYSCTL->RCGCGPIO |= (1<<5);

GPIOF->LOCK = 0x4C4F434B;

GPIOF->CR = 0x01;

GPIOF->LOCK = 0;

GPIOF->DIR &= ~(1<<4)|~(1<<0);

GPIOF->DIR |= (1<<3);

GPIOF->DEN |= (1<<4)|(1<<3)|(1<<0);

GPIOF->PUR |= (1<<4)|(1<<0);

GPIOF->IS  &= ~(1<<4)|~(1<<0);

GPIOF->IBE &=~(1<<4)|~(1<<0);

GPIOF->IEV &= ~(1<<4)|~(1<<0);

GPIOF->ICR |= (1<<4)|(1<<0);

GPIOF->IM  |= (1<<4)|(1<<0);

NVIC->IP[30] = 3 << 5;

NVIC->ISER[0] |= (1<<30);

while(1)

{

// do nothing and wait for the interrupt to occur

}

}

void GPIOF_Handler(void)

{

if (GPIOF->MIS & 0x10)

{

GPIOF->DATA |= (1<<3);

GPIOF->ICR |= 0x10;

}

else if (GPIOF->MIS & 0x01

{

GPIOF->DATA &= ~0x08;

GPIOF->ICR |= 0x01;

*/

}

}

ii)POLLING METHOD:

PROGRAM:

#include "TM4C123GH6PM.h"

int main(void)

{

unsigned int state;

SYSCTL->RCGCGPIO |= 0x20;

GPIOF->LOCK = 0x4C4F434B;

GPIOF->CR = 0x01

GPIOF->PUR |= 0x10;

GPIOF->DIR |= 0x02;

GPIOF->DEN |= 0x12;

while(1)

{

state = GPIOF->DATA & 0x10;

GPIOF->DATA = (~state>>3);

}

}

OUTPUT:

Experiment No. 04 : Implementation of PWM to change duty cycle.

PROGRAM:

#include "TM4C123GH6PM.h"

int main(void)

{

void Delay_ms(int n);

int duty_cycle = 4999;

SYSCTL->RCGCPWM |= 2;

SYSCTL->RCGCGPIO |= 0x20;

SYSCTL->RCC |= (1<<20);

SYSCTL->RCC |= 0x000E0000;

GPIOF->AFSEL |= (1<<2);

GPIOF->PCTL &= ~0x00000F00;

GPIOF->PCTL |= 0x00000500;

GPIOF->DEN |= (1<<2);

PWM1->_3_CTL &= ~(1<<0);

PWM1->_3_CTL &= ~(1<<1);

PWM1->_3_GENA = 0x0000008C;

PWM1->_3_LOAD = 5000;

PWM1->_3_CMPA = 4999;

PWM1->_3_CTL = 1;

PWM1->ENABLE = 0x40;

while(1)

{

duty_cycle = duty_cycle - 10;

if (duty_cycle <= 0)

duty_cycle = 5000;

PWM1->_3_CMPA = duty_cycle;

Delay_ms(100);

}

}

void Delay_ms(int time_ms){

int i, j;

for(i = 0 ; i < time_ms; i++)

for(j = 0; j < 3180; j++)

{}

}

void SystemInit(void)

{

SCB->CPACR |= 0x00f00000;

}

OUTPUT:

Experiment No. 05: Communication through UART

UART TRANSMITTER

PROGRAM:

#include "TM4C123.h"

void Delay(unsigned long counter);

char UART5_Receiver(void);

void UART5_Transmitter(char data);

int main(void)

{

SYSCTL->RCGCUART |= 0x20;

SYSCTL->RCGCGPIO |= 0x10;

Delay(1);

UART5->CTL = 0;

UART5->IBRD = 104;

UART5->FBRD = 11;

UART5->CC = 0;

UART5->LCRH = 0x60;

UART5->CTL = 0x301;

GPIOE->DEN = 0x30;

GPIOE->AFSEL = 0x30;

GPIOE->AMSEL = 0;

GPIOE->PCTL = 0x00110000;

Delay(1);

while(1)

{

UART5_Transmitter('H');

UART5_Transmitter('E');

UART5_Transmitter('L');

UART5_Transmitter('L');

UART5_Transmitter('O');

UART5_Transmitter('\n');

}

}

void UART5_Transmitter(char data)

{

while((UART5->FR & 0x20) != 0);

UART5->DR = data;

}

void Delay(unsigned long counter)

{

unsigned long i = 0;

for(i=0; i< counter; i++);

}

OUTPUT:

UART RECEIVER

PROGRAM:

#include "TM4C123.h"

#include <stdint.h>

#include <stdlib.h>

void Delay(unsigned long counter);

char UART5_Receiver(void);

void UART5_Transmitter(unsigned char data);

void printstring(char *str);

int main(void)

{

SYSCTL->RCGCUART |= 0x20;

SYSCTL->RCGCGPIO |= 0x10;

Delay(1);

UART5->CTL = 0;

UART5->IBRD = 104;

UART5->FBRD = 11;

UART5->CC = 0;

UART5->LCRH = 0x60;

UART5->CTL = 0x301;

GPIOE->DEN = 0x30;

GPIOE->AFSEL = 0x30;

GPIOE->AMSEL = 0;

GPIOE->PCTL = 0x00110000;

Delay(1);

printstring("Hello World \n");

Delay(10);

while(1)

{

char c = UART5_Receiver();

UART5_Transmitter(c);

}

}

char UART5_Receiver(void)

{

char data;

while((UART5->FR & (1<<4)) != 0);

data = UART5->DR ;

return (unsigned char) data;

}

void UART5_Transmitter(unsigned char data)

{

while((UART5->FR & (1<<5)) != 0);

UART5->DR = data;

}

void printstring(char *str)

{

while(*str)

{

UART5_Transmitter(*(str++));

}

}

void Delay(unsigned long counter)

{

unsigned long i = 0;

for(i=0; i< counter; i++);

}

void SystemInit(void)

{

__disable_irq();

SCB->CPACR |= 0x00F00000;

}

OUTPUT:

FACTORIAL IN MASM:

assume cs:code

code segment

start: mov si,000h

mov cl,[si]

mov al,01h

X:MUL cl

DEC cl

JNZ X

int 03h

code ends

end start

Fibonacci by MASM:

assume cs:code , ds:data

data segment

res db 10 dup(10h)

data ends

code segment

start: mov ax,data

mov ds, ax

mov di, offset res / LEA di, res

mov al,00h

mov bl,01h

mov [di],bl

mov res, di

inc di

mov ch,08h

X: mov cl,al

Add al,bl

DAA
mov[di],al

mov res,di

mov bl,cl

inc di

dec ch

jnz X

int 03h

code ends

end start

Invert lower nibble of 45h by MASM:

assume cs:code

code sgment

mov ax,45h

XOR AX,0Fh

mov BX,AX

code ends

end

To evaluate arithmetic exp 3x^3-2x+8 by MASM:

assume cs:code

code segment

start:

mov ax,02h

mov bx,02h

mul bx

mul bx

mov bx,03h

mul bx

mov dx,ax

mov ax,02h

mov cx,02h

mul cx

mov bx,ax

sub dx,bx

mov ax,dx

add ax,08h

int 3h

code ends

end start

To find the LCM of two no's in masm

Registers used: AX, BX, CX, PX

Software used: MASM.

Program:

ascume code

Code segment

start mov ax, 03h

mov bx, 11h

back: mor dx, 0.

mov cx, bx

div bx

mov ax, bx

mov by, dx

Icmp bx, o'

jne back

до : mov ax, 3

mov bx, 11

mul bx

div cx

int 03h

Code ends.

end start

Find the average of 'n' numbers Registers used:

Registers used: ax, bx, cx

Software used: MASM.

Program:

assume cs: code

code segment

start: mov cx,03h

mov bx,cx

mov ax,00h

X: add ax,cx

loop X

div bx

int 03h.

code ends

end start

DELETING R FROM MICROPROCESSOR:

Assume cs:code , ds:data

data segment

str1 db microprocessor$"

len du ($-str1)

res1 db 15 dup(?)

data ends

code segment

start:

mov ax, data

mov ds,ax

mov bl,'r'

mov cx, len

mov SI,offset str1

mov DI,offset resi

loop1:

mov al, [SI]

CMP al,bl

JE loop2

mov [DI], al

INC DI

loop2:

INC SI

DEC CX

JNZ loop1 mou bl,"$"

mov SI,offset res1

mov dx,Oh

last1:

CMP bl, [SI]

JE last

INC dx

INC SI

JMP last1

last:

mov ah, 09h

int 21h

int 3h

code ends

end start

TO FIND POSITIVE/NEGATIVE NUMBERS PRESENT IN THE GIVEN ARRAY

data segment

n1      db   56h, 49H, 33h

data ends

code segment

assume cs: CODE, ds:data

start:	mov ax, data

mov ds, ax

xor ax, ax/mov ax,0000h

mov SI, OFFSET n1

MOV DX, 0000H

MOV BX, 0000H

MOV CX, 0003H

bACK:	MOV AL, [SI]

ROL AL, 01H

JC     X

INC BX

JMP Y

X:              INC DX

Y:               INC SI

DEC CX

JNZ    BACK

INT 03H

code ends

end start